GUIDELINE NO. 7

Fences

All Forms must include the following information:

Chain link, Vinyl and Free-Standing hog wire fence are prohibited.

Picture or drawing of fence type. (See Rules and Regulations for the attached Fence Exhibits for acceptable styles).

Site Plan: A site plan denoting the location of the fence must accompany the Form. Fences shall not be located closer to any street than the rear edge of the home. However, on corner lots, the fence shall not be closer to any side street than the building line of the lot (minimum 15 ft). No fencing may impede line of sight for motorists. Setbacks for golf course lots may be different. It is the property owner’s responsibility to know where their property line sits by obtaining a professional survey before installing a fence. Existing fences aren’t necessarily indicative of a property line.

Obtain a Plat Map: If available, acquire a plat map of your property from your local government or homeowner's association. This map will provide accurate information about your property lines and boundaries.

Consult a surveyor: If there is any uncertainty or question about your property lines, it is advisable to consult a licensed surveyor. A surveyor can accurately determine the boundaries of your property and provide guidance on fence placement.

Color: - Wood fences must be natural stained. Wood fences need to be re-stained every year or two. Inspect for rot and bug infestation. Wrought iron/aluminum fences must be black properly maintained and painted as necessary. All nails, screws or fasteners shall be aluminum or hot-dipped galvanized. Cedar elements on your property, such as fences or siding, you are welcome to let them age naturally without intervention.

Dimensions: - Maximum heights are noted above. The maximum span between posts shall be ten feet (10’). The minimum post size shall be four inches by four inches (4” x 4”) and must have two inches by eight inches (2” x 8”) rails or three (3) two inch by six-inch (2” x 6”) rails per section. Maximum width of gate opening is 72”.

Crossbeam - Except for exterior lots backing up to non-residential property, crossbeam structure shall not be visible from any street (must face inside toward yard).

Posts: All posts shall be anchored in concrete.

All fences constructed on any lot shall be of uniform style and construction, including fencing used for dog runs. A form must be submitted for all dog runs. Dog runs must meet all fence guidelines.

Golf Course Lots - Fences must be black iron/aluminum. No wood fences.

The following types of fencing may be approved for installation:

Four-foot (4’) picket fences, straight, scalloped, or inverted scalloped with an approved tip style (See Exhibit “A” at end of document). Materials must be cedar, cypress, or No. 2 grade or better pressure treated wood.

Four-foot (4’) picket fences, straight, scalloped, inverted scalloped or staggered with an approved tip style (See Exhibit “B”). Materials must be cedar, cypress, or No. 2 grade or better pressure treated wood.

Maximum five foot (5’) wrought iron or aluminum style fence with an approved tip style (See Exhibit “D”)

Six-foot (6’) privacy with flat, scalloped, or inverted scalloped tops and post detail as noted. (See Exhibit “C”). Materials must be cedar, cypress, or No. 2 or better pressure treated wood.

Six-foot (6’) privacy with scalloped tops and post detail as noted. (See Exhibit “C” - includes top to match picket fence profile). Materials must be cedar, cypress, or No. 2 or better pressure treated wood.